CSE – 313s

Digital Systems Testing and Verification

Final Project Report

SPI Master with APB Interface

Team-19

| Name | ID | Specialization |
| --- | --- | --- |
| Ziad Ahmed Orabi | 2200399 | CSE |
| Mathew Mina Youssef Aziz | 2300910 | CSE |
| Arsany Hany Anwar | 2200655 | CSE |
| Andrew Ehab Tharwat | 2200249 | CSE |
| Mina Fathy Labib | 2200840 | ECE |
| Radwa Yasser Ahmed | 2200239 | CSE |
| George Safwat Wasfy | 2200365 | ECE |

- Table of Contents

1	Introduction	1

1.1	Project overview	1

1.2	Verification methodology	1

1.3	Report rganization	1

2	Design Under Test	2

2.1	Architecture	2

2.2	FSM and transfer behavior	2

2.3	Slave select and FIFO control	2

2.4	Interrupt logic	2

3	Verification Environment	4

3.1	Testbench architecture	4

3.2	Environment components	4

3.3	Coverage model	4

3.4	Regression infrastructure	5

3.5	Parallel regression runner	6

4	Simulation Results	7

5	Bugs Found	8

6	Conclusion	9

# Introduction

## Project overview

This report documents the verification environment developed for the SPI Master Controller IP as part of the Digital Design Verification final project at Ain Shams University, Spring 2026. The DUT mimics the register layout of production-grade SPI masters found in ARM MCUs, making the verification lessons directly transferable to industry practice.

The objective of this project is to design and implement a comprehensive SystemVerilog verification environment capable of detecting functional bugs in the DUT against a defined specification.

## Verification methodology

The verification methodology adopted is SystemVerilog-only, utilizing constrained-random stimulus, functional coverage, a reference model-based scoreboard, and SystemVerilog Assertions (SVA). Critically, the verification environment must remain functional when the golden RTL is replaced by any of the 28 buggy variants without modification.

## Report organization

The report is organized as follows: Section 2 describes the DUT architecture and key requirements, Section 3 details the verification environment components, Section 4 presents simulation results, Section 5 discusses bugs found, and Section 6 concludes with lessons learned.

# Design Under Test

## Architecture

The DUT is an SPI Master Controller IP implemented in SystemVerilog. It is structured as a three-file hierarchy: spi_master.sv serves as a thin top-level wrapper instantiating two sub-blocks; apb_regfile.sv, which implements the APB v2.0 slave interface, nine programmable registers, TX and RX FIFOs (8 entries, 32-bit wide each), and interrupt logic; and spi_core.sv, which implements the SPI shift FSM, SCLK generation, and MOSI drive logic.

The DUT operates in a single PCLK domain, with SCLK derived as a divided and gated version of PCLK. Active-low PRESETn with synchronous deassertion is assumed, and all flops carry explicit reset values, eliminating X-optimism.

## FSM and transfer behavior

The spi_core FSM operates across four states: S_IDLE, S_SHIFT, S_FINISH, and S_GAP. The S_GAP state activates only when DELAY ≠ 0 and another TX word is waiting, inserting idle cycles equal to (2 + DELAY) × (DIV+1) PCLK cycles between consecutive transfers. For CPHA=0, the first bit is presented on MOSI before the first SCLK edge; for CPHA=1, the first bit is launched on the first SCLK edge. Configuration registers cfg_mode, cfg_lsb_first, cfg_width, and cfg_clk_div are latched into transfer-local xfer_* registers at the S_IDLE to S_SHIFT transition per R25, meaning mid-transfer register writes take effect only on the next transfer.

## Slave select and FIFO control

SS_n is driven combinationally by apb_regfile using the formula ~ss_en | ss_val. The core observes SS_n through ss_n_drive and starts a transfer only when at least one SS lane is driven low, and the TX FIFO is non-empty. Both the TX and RX FIFO pointers are reset not only on PRESETn deassertion but also whenever CTRL.EN is deasserted, ensuring a clean state on every enable cycle.

## Interrupt logic

The five interrupt sources are TX_EMPTY (bit 0), RX_FULL (bit 1), TX_OVF (bit 2), RX_OVF (bit 3), and TRANSFER_DONE (bit 4). The IRQ output is the bitwise AND of INT_STAT and INT_EN. The STATUS register is read-only, TX_DATA is write-only with reads returning zero, and INT_STAT implements W1C semantics per R18.

# Verification Environment

## Testbench architecture

The verification environment is implemented in plain SystemVerilog following the SV-only track defined in the grading interface. The testbench top-level module, tb_top, instantiates the DUT through a wrapper module dut_wrapper, which exposes the APB and SPI interfaces and binds the SVA module into the DUT hierarchy. Test dispatch is controlled via the +TESTNAME=<name> plusarg, with the Makefile passing both +TESTNAME and +UVM_TESTNAME for compatibility.

## Environment components

The environment consists of four principal components: APB Master BFM, SPI Slave BFM, Reference Model, and the Coverage Collector.

The APB Master BFM drives the APB interface to perform register read and write transactions, abstracting protocol timing from the test layer. The SPI Slave BFM monitors and drives the SPI bus, capturing MOSI data and responding on MISO according to the configured mode, width, and bit order.

The reference model maintains a predicted state of the DUT, including TX and RX FIFO occupancy, interrupt status, and expected RX data for each transfer, emitting [SCOREBOARD_ERROR] messages on any mismatch between prediction and observation. The reference model enforces a strict contract: predict_transfer() must be called before every TX push, and verify_rx_drain() must be called after every RX read, maintaining queue balance at all times.

## Coverage model

The coverage collector implements fourteen covergroups targeting the full configuration space, including SPI mode, transfer width, bit order, CLK_DIV corners, FIFO occupancy, interrupt sources, W1C behavior, loopback, delay, and slave select control. Notably, cg_irq covers the W1C race condition defined in R18, where a hardware event sets a bit on the exact same cycle that software writes a W1C clear, the specification requires the set to win.

APB protocol correctness is captured in cg_apb_protocol, which enforces that PSLVERR remains low and PREADY remains high on every access. The cg_ctrl_en covergroup specifically targets the non-trivial case where EN=0 must override an active SS_CTRL configuration, closing the cp_ss_high bin. Reserved address accesses at offsets ≥ 0x24 are covered by cg_reserved, verifying that reads return zero and writes are silently ignored per R23.

In summary: Thirty Seven SVA assertions are bound into the DUT hierarchy via u_wrap.u_dut.u_regfile and u_wrap.u_dut.u_core, checking properties including SCLK idle level, SS_n behavior when EN=0, and BUSY assertion during active transfers. These hierarchical paths are also used for backdoor register reads during reset value verification.

## Regression infrastructure

The verification flow is driven by a single Makefile targeting QuestaSim. It supports four top-level knobs overridable from the command line: SIMULATOR, SEED, TEST, and WAVES, all defined with “=” so any command-line value takes precedence over the default.

Source files are organized into four groups: TB_SRCS containing the APB Master BFM, SPI Slave BFM, and tb_top; ENV_SRCS containing the reference model and coverage collector; SEQ_SRCS containing the stimulus library; and ASSERT_SRCS containing the SVA assertions. The DUT source list defaults to the three golden RTL files and is overridden by the grader via DUT_SRCS to point at buggy variants. A compatibility shim EFF_DUT_SRCS selects DUT_SRC if set, otherwise falling back to DUT_SRCS.

The compile target creates a fresh Questa work library and compiles all sources in dependency order: interfaces, DUT, wrapper, environment, sequences, assertions, then testbench, all in a single vlog invocation with coverage instrumentation enabled. The run target depends on compile and launches vsim in batch mode, passing +TESTNAME and +SEED as plusargs, saving a per-test coverage database on exit.

The regression is defined as 11 tests × 5 seeds = 55 total simulation runs, each redirected to a per-test per-seed log file under build/. After all runs complete, the regress target merges all .ucdb coverage databases into a single build/merged.ucdb via vcover merge. The cov target then generates coverage_report.txt from the merged database, failing with a clear error message if the merged database does not exist, ensuring clean CI behavior.

## Parallel regression runner

To accelerate regression turnaround, a Python script `run_parallel.py` was developed to execute multiple test/seed combinations concurrently using a thread pool of up to 10 parallel workers. The script is driven entirely from the command line, accepting five flags: `--tests`, `--seeds`, `--workers`, `--compile`, and `--makefile-dir`, making it flexible for both full regression runs and targeted single-test debugging.

The script begins by optionally invoking `make compile` once before any tests start, streaming compiler output to the terminal in real time. It then constructs the full set of (test × seed) job combinations and submits them to a `ThreadPoolExecutor`. Each worker calls `make run TEST=… SEED=… WAVES=0`, captures stdout and stderr, saves them to a per-test per-seed log file under `build/`, and classifies the result as pass or fail by scanning for known Questa and UVM keywords. Failure markers are checked first — `TEST_FAILED`, `** Error`, `ASSERTION_FAILED` — before checking for pass markers, with the make exit code used as a fallback if neither pattern is found.

Upon completion, the script prints an aligned summary table showing pass/fail status, elapsed time, and log path for every run. Failed runs are accompanied by up to five extracted error lines from the log, eliminating the need to open individual log files for triage. The script also reports wall-clock duration against the sequential equivalent, producing a speedup factor that quantifies the benefit of parallelism.

Finally, all Questa `.ucdb` coverage databases produced during the regression are merged into `build/merged.ucdb` via `vcover merge`, followed by a `vcover report` call that generates `coverage_report.txt`. The script exits with code 0 if all runs pass and code 1 if any fail, making it directly compatible with CI pipelines.

# Simulation Results

# Bugs Found

# Conclusion